WATER UNDER THE BRIDGE

Did you know that even if fish manage to pass through poorly designed culverts, they may be injured or weakened by strong currents?

Home Sweet Home

A healthy fish habitat not only reflects on the health of the fish themselves, but also on the health of the mini-ecosystems on the golf courses.  Fish habitat are the are the parts of the aquatic environment that fish depend on, directly or indirectly in order to carry out their life processes.  They are the areas where spawning, rearing, food supply and migration occurs.  They are the areas necessary for the fish to live out their lives successfully.

The streams and rivers that travel through the golf courses, and the wetlands that occupy the lowlands are prospective habitat areas.  These habitats meet the needs for the fish for quality water, food and shelter.  If these are not met due to damage or loss of habitat, their populations will decline or eventually disappear.  The Department of Fisheries and Oceans (DFO) applies the principle of  “No Net Loss” to proposed projects and works.  It is important to protect fish and their habitat.

On many golf courses, stream crossings are common.  However, an improperly designed, built and maintained stream crossing can negatively impact fish and fish habitat.  The causes of most of these problems involve:

- Blocked migration due to design of the crossing.

- Sediment loading into the water ways.

- Loss of vegetation along and in the water way.

- Introduction of deleterious substances during construction.

Culvert or Bridge?

Different structures of bridges and culverts have been designed for stream crossings.  Either one can be used for crossing, but they each have their own advantages and disadvantages.  However bridges are the favored structure for fish and habitat protection because they retain the existing bank structure and riparian vegetation while there is potential loss with the use of culverts.

Bridges can be designed to minimize the effects on the environment.  Piers and footings on the bridge can be constructed in such a way that they are outside of the stream and above the high water line.  The effects on the riparian zone and river banks are minimized by such a design.  Other measures that should be implemented into the design of the bridge includes avoiding the use of concrete aprons and avoid extending bank protection beyond the necessary distance that the bridge spans.  During bridge construction, in-stream work should be separated from the water with a cofferdam (a watertight enclosure pumped dry to permit work below the waterline). In other words do not disturb more than is absolutely necessary.

Culverts are ducts used to allow the passage of fish and water under a road or through a dike.  There are different types and sizes for various situations and certain factors should be taken into consideration with respect to fish passage.  The velocity of the currents generated from passage through a culvert should not exceed the maximum prolonged swimming capacity of fish.  Some fish may be injured or weakened by strong currents thus making them susceptible to diseases and predators.  If more energy than normal is used to move through the culvert, then less energy will be allocated to reproduction.  The diameter and opening of the culvert should not restrain the width of the stream bed.  If it does then debris may build up and make the crossing impassable for fish.  The best culverts for fish passage are the open-bottomed box or arch culvert.  Both of these are similar to a bridge in that they retain the natural stream bottom.  The closed-bottom variations do not maintain the natural stream substrate beneath the culvert.

The physical presence of stream crossings or the construction associated with the structure affect fish and their habitat.  As a result of a bridge or culvert, there is alteration and even loss of fish habitat within the imprint of the structure, increased sediment loading, impaired fish passage, and changes to channel morphology.  Proper planning of what type of crossing can not only protect fish and their habitat, but also the wildlife communities local and downstream.

Works Cited:

Fisheries and Oceans and Manitoba Natural Resources.  Manitoba stream crossing guidelines for the protection of fish and fish habitat.  1996.

- Transportation Association of Canada.  Transportation construction and maintenance and the protection of fish habitat.  1999.